ISOLATED WALLS

Update blue text to suit project

  - Description

- Scope

- All studio/cinema/auditorium walls shall fully isolated from the main building structure to prevent the transmission of sound and vibration to the building structure. Use isolation products supplied by Mason U.K. Ltd or equal and approved.

Mason UK Ltd: Tel 01252 716610 or www.mason-uk.co.uk.

- All isolation materials shall be supplied by the same manufacturer.

- Any variance or non-compliance with the specification shall be corrected by the contractor in an approved manner.

  - Submittal data requirements

A.  The manufacturer of the vibration isolation shall provide submittals for products as follows:

- Catalogue data sheets.

- Specifications of products, to include deflection range, load range and calculated deflection and natural frequency based on specific loadings. All calculations for rubber/neoprene products to be supported by indicative dynamic response curves verified by an independent source.

Specification

A. Wall supports

- Walls must either sit on an isolated floating floor or on a dedicated isolated wall support. The isolated wall support must have a similar frequency response to adjacent acoustic floors. The maximum natural frequency of the wall supports shall be 12Hz. The design of the wall support is to be specific to each wall to ensure correct isolation.

- All metal parts used in isolated wall footings to be galvanised against corrosion.

- All isolated wall supports shall be SWW as manufactured by Mason U.K. or equivalent.

B. Wall ties

- If required structurally, walls must be supported by acoustic wall ties. All wall ties are to use Low Dynamic Stiffness elastomers. The wall ties are to provide a maximum natural frequency of 12Hz. The wall ties are to use low dynamic stiffness natural rubber as the isolating element.

- Wall ties are to be supported by product datasheets detailing maximum axial rating and dynamic characteristics.

- Wall ties shall be type WIC as manufactured by Mason U.K. or equivalent.

C. Wall head restraint

- The wall head shall be resiliently supported and sealed against the soffit by an acoustic head-restraint rail. The wall head shall not physically touch the soffit.

- The head restraint rail shall support the wall head via closed cell low-density foam with a minimum thickness of 10mm. The rail shall comprise of a 90° angle fixed to the soffit. The design of the rail shall allow space for a non-hardening mastic bead, used to fill gaps caused by variation in wall flatness.

- Wall head restraint channels shall be AB-716 as manufactured by Mason U.K. or equivalent.